Tibboh APN Settings for Sony-Ericsson X1
	Click Start menu and choose Settings
	[image: image1.png]£ Today
8. Ele Explorer

8 Widknowledge Mobile

£ Intemnet Enplorer
Opera 9.5 Beta
T word Mobile

& Messaging
B solitaire
© Windows Media

R programs
) settings







	Choose the Connections tab and the Connections icon
	[image: image2.png]





	Under My ISP heading (or Internet Contract, or My Work) click on Add a new modem connection
	[image: image3.png]Sel

g5

Connections

My ISP
dd a new modem connection

Manage existing connectons

My Work Network
£dd 2 new modem connection
‘dd 2 new VPN server connection

Set up my proxy server







	Under Enter a name for the connection enter:

tibboh

Under Select a modem choose:

Cellular Line (GPRS, 3G)
Click Next


	[image: image4.png]elllar Line (GPRS, 3G)

Next







	Under Access Point Name (APN) enter:
· tibboh.co.uk

and click Next
	[image: image5.png]





	Under Username enter:

· tibboh
Under Password enter:

· internet

Domain stays clear

Click Finish
Click ok to exit.


	[image: image6.png]Finish







Troubleshooting:
If your phone doesn’t initialise a connection with tibboh check the following:

· You turned on your phone (under the Comm Manager: Start > Settings > Connections > Comm Manager)
· Sometimes the HSPDA is a problem to get a connection, disable it under Start > Settings > Connections > Advanced Connections and restart your device.

